KOMPOSOS

PFAS Compliance

Screening Report

Prepared for:

[Client Company Name]

Portfolio: Li-Ion Cell Materials (47 substances)

Report Date: March 24, 2026

Report ID: PFAS-2026-0324-0047

CONFIDENTIAL  |  Prepared by KOMPOSOS Compliance Engine v3.1  |  Apache 2.0 / Commercial

# 1. Executive Summary

| Materials Screened | PFAS Detected | Replacements Found | Compliance Risk |
| --- | --- | --- | --- |
| 47 | 3 | 9 candidates | HIGH |

This report screens 47 materials in [Client]'s Li-Ion cell bill of materials against 35 curated PFAS substances mapped to EU, US EPA, and Stockholm Convention regulations. Three materials contain PFAS compounds subject to the EU PFHxA restriction effective October 4, 2026.

For each flagged material, this report provides: the specific PFAS substance detected, the applicable regulation and deadline, scored replacement candidates with application-specific compatibility analysis, and a full audit trail traceable to source data.

# 2. Regulatory Timeline

| Date | Regulation | Scope | Status |
| --- | --- | --- | --- |
| Oct 4, 2026 | EU PFHxA Restriction (2021/1297) | Manufacturing + import ban | 190 DAYS |
| Jul 1, 2026 | US EPA TRI Reporting (9 new PFAS) | Mandatory reporting | 99 DAYS |
| 2027 | EU Universal PFAS Restriction (proposed) | 5,000+ substances | PENDING |

# 3. Portfolio Screening Results

## 3.1 PFAS Detection Summary

| Material | CAS Number | Function | PFAS Substance | Verdict |
| --- | --- | --- | --- | --- |
| PVDF (Kynar 761) | 24937-79-9 | Cathode binder | Polyvinylidene fluoride | PFAS DETECTED |
| FEP coating (Teflon FEP) | 25067-11-2 | Separator coating | Fluorinated ethylene propylene | PFAS DETECTED |
| PTFE gasket seal | 9002-84-0 | Cell seal | Polytetrafluoroethylene | PFAS DETECTED |
| NMC811 (cathode active) | -- | Cathode | -- | PFAS-FREE |
| LiPF6 (electrolyte salt) | 21324-40-3 | Electrolyte | -- | PFAS-FREE |
| ... (42 more materials) | -- | -- | -- | PFAS-FREE |

# 4. Replacement Analysis

## 4.1 PVDF Binder Replacement

PVDF (polyvinylidene fluoride) is the most critical finding. It is the primary cathode binder in the current cell design. Three replacement candidates were screened for compatibility with the NMC811 cathode, LiPF6 electrolyte, and Cu current collector.

| Replacement | Adhesion | Electrolyte | Thermal | Cathode | Overall | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| CMC/SBR blend | 0.82 | 0.88 | 0.71 | 0.85 | 0.84 | VALIDATED |
| PAA (polyacrylic acid) | 0.91 | 0.76 | 0.83 | 0.79 | 0.80 | VALIDATED |
| Alginate (Na-alginate) | 0.74 | 0.69 | 0.65 | 0.72 | 0.71 | CAUTION |

Recommendation: CMC/SBR blend is the highest-scoring replacement (0.84 overall) with validated compatibility across all four domains. PAA is a strong secondary option with superior adhesion (0.91) but reduced electrolyte stability. Alginate scores below the 0.75 threshold for thermal compatibility and requires further testing before qualification.

## 4.2 Compatibility Provenance (CMC/SBR)

Every score above traces to specific source data. Below is the provenance chain for the CMC/SBR cathode compatibility score of 0.85:

| Property | Value | Source | Contribution |
| --- | --- | --- | --- |
| Binding energy (NMC811) | 1.8 J/m² (vs PVDF: 2.1 J/m²) | mp-756938, mp-25747 | 0.82 (86% of PVDF) |
| CTE mismatch | 4.2 ppm/K (within 10 ppm limit) | mp-756938 | 0.91 (pass) |
| Electrochemical window | 0.1 - 4.5 V (covers NMC range) | DOI: 10.1021/acs... | 0.88 (pass) |
| Slurry processability | Water-based (vs NMP for PVDF) | Industry standard | 0.79 (advantage) |

Note: CMC/SBR uses water-based slurry processing, eliminating NMP solvent (a regulated toxic substance). This represents an additional compliance advantage beyond PFAS replacement.

# 5. Recommended Action Plan

| Priority | Action | Timeline | Risk if Delayed |
| --- | --- | --- | --- |
| P0 | Qualify CMC/SBR binder on NMC811 cathode line | Weeks 1-8 (by June 2026) | Cell production halt Oct 2026 |
| P1 | Source PTFE-free gasket seal alternative | Weeks 1-4 | Cell assembly non-compliant |
| P1 | Evaluate non-fluorinated separator coating | Weeks 2-6 | Separator supply chain risk |
| P2 | Document full BOM audit trail for EU regulator | Weeks 4-8 | Compliance documentation gap |
| P3 | Monitor EU universal PFAS restriction (2027) scope | Ongoing | Future supply chain disruption |

# 6. Methodology & Limitations

This screening was performed using the KOMPOSOS Compliance Engine v3.1. Materials were screened against a curated database of 35 PFAS substances with CAS number verification. Compatibility scores are computed via structured interpolation over 103,644 materials from the Materials Project database, with provenance tracing to specific source entries.

Compatibility scores represent multi-domain analysis: each replacement is scored against the cathode, electrolyte, thermal environment, and current collector simultaneously. Scores above 0.75 are classified VALIDATED; 0.60-0.75 CAUTION; below 0.60 VETOED.

Limitations: (1) Compatibility scores are interpolated from known materials data, not ab initio quantum chemistry calculations. Accuracy has been validated via leave-one-out testing (voltage error 1.6-7.2%, crystal structure 23/23 correct). (2) Scores do not replace physical qualification testing. (3) PFAS substance list covers the 35 most regulated compounds; the broader EU restriction (2027) may add thousands more.

# 7. Audit Certificate

| Report ID | PFAS-2026-0324-0047 |
| --- | --- |
| Screening Engine | KOMPOSOS Compliance Engine v3.1 |
| Materials Screened | 47 (full Li-Ion cell BOM) |
| PFAS Database | 35 substances (CAS-verified, EU/US/Stockholm mapped) |
| Materials Database | 103,644 entries (Materials Project + 169 curated) |
| Verification Method | Dual-engine (categorical + ZFC set-theoretic) with provenance |
| Validation Status | Leave-one-out validated: 1.6-7.2% voltage error, 23/23 structure |
| Test Suite | 1,423 automated tests passing |
| Generated | March 24, 2026 at 14:32 UTC |
| Prepared by | James Ray Hawkins ¦ jhawk314@gmail.com ¦ KOMPOSOS |

END OF REPORT

This report is generated by automated screening software and does not constitute legal, regulatory, or engineering advice. Physical qualification testing is required before production changes. Consult legal counsel for regulatory compliance decisions.